Spacecraft Timekeeping Complete Formatted Execution Plan

Same content retained, reformatted into one consistent phase-based project document

# 1. Purpose

This document keeps the full execution plan and formats the simulation, tabletop hardware demonstrator, RC-car demonstrator, and high-fidelity extensions as one consistent project. The RC car is treated as a spacecraft bus analogue, not as a separate project.

# 2. Immediate Scope Decision

The first milestone must prove the timing system, not every possible aerospace subsystem. Phase 1 therefore remains a pure Python timing simulator. Advanced tools and protocols are preserved in the plan but moved to later phases.

| Keep in Phase 1 | Move to Later Phases |
| --- | --- |
| Rubidium clock noise model | GMAT |
| Simulated GNSS time measurements | GPS-SDR-SIM |
| Kalman clock synchronization engine | GNSS-SDR |
| PPS distribution and skew model | TWSTFT |
| Ground UTC comparison using simple reference time | CCSDS packet timing |
| Four validation plots | Orekit |
| Pure Python repository structure | Transponder model |

# 3. Frozen Block Diagram

Use the same block diagram in the Functional Flow Report, Python Guide, Final Presentation, and RC-car demonstrator section.

GPS Satellites ¦ v GNSS Antenna ¦ v TR-2S Receiver ¦ v GNSS Time + PPS ¦ v Raspberry Pi / Mini PC ¦ v Time Synchronization Software ¦ v Master Time Output ¦ v PPS Distributor (Software) ¦ v Simulated TMTC / AOCS / Payload / OBC ¦ v Telemetry ¦ v Laptop Dashboard Physical carrier for hardware demonstrator: RC Car Chassis

# 4. Phase Plan

| Phase | Name | Goal | Included Work | Output |
| --- | --- | --- | --- | --- |
| Phase 1 | Pure Python Timing Simulation | Prove GNSS + Rubidium + Kalman Filter = Stable Master Time. | Rubidium clock model; simulated GNSS measurements; Kalman synchronization engine; PPS distribution simulation; Ground UTC comparison. | Four plots and a working simulation. |
| Phase 2 | Tabletop Timing Demonstrator | Prove the architecture with real GNSS and no vehicle motion. | TR-2S, GNSS antenna, Raspberry Pi, time synchronization software, dashboard, PPS monitoring. | Live dashboard showing GNSS status, PPS lock, mode, and time offset. |
| Phase 3 | RC-Car Spacecraft-Bus Demonstrator | Mount the same timing avionics on an RC car and observe real GNSS behavior during motion. | RC car chassis, battery, TR-2S, GNSS antenna, Raspberry Pi, logger, dashboard telemetry. | Mobile timing demonstration and field logs. |
| Phase 4 | Ground Validation | Add more realistic ground comparison and holdover analysis. | Propagation delay, one-way time transfer, Ground UTC comparison, holdover analysis. | Ground validation plots and error budget. |
| Phase 5 | High-Fidelity Timing Model | Add research-grade tools after the architecture is proven. | GNSS-SDR, GPS-SDR-SIM, GMAT/Orekit, TWSTFT, CCSDS packet timing, transponder. | Advanced simulation and research extension. |

# 5. Phase 1 Repository Structure

spacecraft_timekeeping/ rb_clock.py gps_receiver.py kalman_clock.py pps_distributor.py main_simulation.py plots/

# 6. Phase 1 Module Definitions

| Module | Responsibility | Minimum Output |
| --- | --- | --- |
| rb_clock.py | Generate Rubidium clock bias, drift, and drift-rate over a 24-hour simulation. | Raw clock error time series. |
| gps_receiver.py | Generate simulated GNSS timing measurements using true time plus Gaussian noise. | Noisy GNSS measurement time series. |
| kalman_clock.py | Fuse Rubidium prediction and GNSS measurements into corrected Master Spacecraft Time. | Corrected clock error time series. |
| pps_distributor.py | Apply per-subsystem PPS cable delay and jitter to TMTC, AOCS, Payload, and OBC. | PPS arrival time and skew values. |
| main_simulation.py | Run the complete 24-hour Phase 1 simulation and save plots. | Four plots and printed validation summary. |

# 7. GNSS Measurement Model for Phase 1

Do not use GNSS-SDR or GPS-SDR-SIM in Phase 1. Use a direct timing measurement model:

gps_time = true_time + np.random.normal(0, 30e-9)

This is enough to test synchronization logic before adding hardware or SDR complexity.

# 8. Required Phase 1 Plots

| Plot | Title | X Axis | Y Axis | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Raw Rubidium Drift | Time | Clock Error | Show how the onboard clock drifts without correction. |
| 2 | Kalman Corrected Error | Time | Clock Error | Show improvement after GNSS correction. |
| 3 | PPS Skew | Subsystem | PPS Arrival Error | Compare timing received by TMTC, AOCS, Payload, and OBC. |
| 4 | Ground UTC Offset | Time | Spacecraft Time - Ground UTC | Show final spacecraft time error against ground reference. |

# 9. Phase 2 Tabletop Timing Demonstrator

The first hardware step should not move. Place the TR-2S receiver, GNSS antenna, and Raspberry Pi on a table. Run the time synchronization software, dashboard, and PPS monitoring. This proves the architecture before adding mobile-platform complications.

| Hardware | Role |
| --- | --- |
| TR-2S Receiver | Provides real GNSS time and PPS. |
| GNSS Antenna | Receives satellite signals. |
| Raspberry Pi 5 / Mini PC | Runs GNSS reader, Kalman filter, PPS logic, logger, and dashboard server. |
| Battery Pack or Stable Power Supply | Provides clean power for the receiver and computer. |
| WiFi Dongle / WiFi | Connects Pi dashboard to laptop. |
| Laptop Dashboard | Displays GNSS status, PPS status, mode, and clock offset. |

# 10. Phase 3 RC-Car Spacecraft-Bus Demonstrator

The RC car is the spacecraft bus analogue. It carries the avionics. It does not change the timing architecture. After tabletop testing works, mount the same system on the RC car and drive around a parking lot, campus, or office area.

| Spacecraft-Term Concept | RC-Car Demonstrator Equivalent |
| --- | --- |
| Spacecraft bus | RC car chassis carrying avionics. |
| GNSS antenna | GNSS antenna mounted on the car or test table. |
| Space-qualified GNSS receiver | TR-2S receiver used as timing receiver. |
| Onboard computer | Raspberry Pi 5 or mini PC. |
| Time synchronization engine | Python synchronization software running on the Pi. |
| PPS distribution system | Software PPS distributor and monitoring logic. |
| TMTC, AOCS, Payload, OBC | Simulated subsystem processes. |
| Ground station / MOC display | Laptop dashboard over WiFi. |

# 11. RC-Car Demonstrator Observables

Satellite visibility.

GNSS quality.

PPS stability.

Time offset.

Telemetry continuity.

Holdover behavior during simulated GNSS loss.

Recovery behavior when GNSS is restored.

# 12. Hardware Requirements

| Required Hardware | Optional Hardware |
| --- | --- |
| Raspberry Pi 5 or mini PC | IMU |
| TR-2S receiver | Camera |
| GNSS antenna | Telemetry radio |
| Battery pack | Real Rubidium source |
| WiFi dongle or onboard WiFi | Vibration-isolated mounting |

# 13. Raspberry Pi Responsibilities

GNSS data reader.

Kalman filter.

PPS distribution logic.

Logger.

Dashboard server.

# 14. Dashboard Demonstration Script

| State | Operator Action | Dashboard Output |
| --- | --- | --- |
| Normal | Antenna connected and satellites visible. | GPS Satellites Visible around 8 to 12; Master Clock Error in ns range; PPS Status = Locked. |
| Holdover | Unplug or shield the GNSS antenna. | GNSS Lost; Mode = Holdover; Rubidium Active; clock error slowly increasing. |
| Recovery | Reconnect or uncover the antenna. | GNSS Acquired; Resynchronizing; Locked. |

If a real Rubidium source is not available, Phase 1 and the first hardware demonstrator should use the software Rubidium model inside the Pi. The goal is to prove synchronization logic first.

# 15. Weekly Execution Sequence

Freeze the block diagram. No more architecture changes until Phase 1 plots are generated.

Create the spacecraft_timekeeping repository with the five Phase 1 modules and plots folder.

Implement rb_clock.py and gps_receiver.py only. Generate clock drift over 24 hours.

Add kalman_clock.py. Generate raw error versus corrected error.

Add pps_distributor.py. Generate the PPS skew plot.

Add Ground UTC comparison and finish all four Phase 1 plots.

Move code to Raspberry Pi.

Read TR-2S GNSS data and PPS status.

Run the dashboard from the Pi and view it on the laptop.

Mount the same working system on the RC car.

Field test in a parking lot, campus, or office area.

# 16. Functional Flow Document Update

Replace the development sequence with this phase sequence:

Phase 1: Pure Python Timing Simulation - Rubidium clock model, simulated GNSS measurements, Kalman synchronization engine, PPS distribution simulation, and Ground UTC comparison.

Phase 2: Tabletop Timing Demonstrator - TR-2S, GNSS antenna, Raspberry Pi, dashboard, and PPS monitoring.

Phase 3: RC-Car Spacecraft-Bus Demonstrator - same timing system mounted on an RC car chassis.

Phase 4: Ground Validation - propagation delay, one-way time transfer, Ground UTC comparison, and holdover analysis.

Phase 5: High-Fidelity Timing Model - GNSS-SDR, GPS-SDR-SIM, GMAT/Orekit, TWSTFT, and CCSDS packet timing.

# 17. Python Guide Update

| Current Content | Action |
| --- | --- |
| GNSS-SDR and GPS-SDR-SIM setup | Keep, but move to Phase 5 appendix. |
| GMAT/Orekit trajectory generation | Keep, but move to Phase 5 appendix. |
| TWSTFT and transponder model | Keep, but move to Phase 5 appendix. |
| CCSDS packet timing | Keep, but move to Phase 5 appendix. |
| SimulatedGPSReceiver | Promote to main Phase 1 GPS model. |
| main_simulation.py | Make it run without external tools. |

# 18. Literature Survey Reference Alignment

| Project Element | Key References | Reason |
| --- | --- | --- |
| Rubidium / clock model | R1, R2, R5 | Clock noise, drift, and Allan deviation validation. |
| GNSS timing and PPS | R6, R10, R11, R12 | GNSS timing behavior, PPS measurement, and receiver timing context. |
| Kalman synchronization | R13, R15, R17, R18 | Clock bias/drift estimation and process-noise modeling. |
| PPS distribution | R6, R7, R8, R9 | PPS accuracy, onboard time access, and mission timekeeping architecture. |
| Ground UTC validation | R30, R31, R32 | UTC handling, ground comparison, and ISRO/NavIC timing context. |
| Advanced extensions | R19-R29 | TWSTFT, transponder, CCSDS, TMTC, and AOCS timing extensions. |

# 19. CEO Milestone Statement

Milestone 1, planned for 4 to 6 weeks, demonstrates GNSS/Rubidium synchronization, Kalman correction, PPS distribution, and Ground UTC comparison in pure Python.

Milestone 2 demonstrates the same architecture on a tabletop using real GNSS, TR-2S, Raspberry Pi, PPS monitoring, and a dashboard.

Milestone 3 mounts the proven timing system on an RC car platform to show real motion, real GNSS data, and live telemetry.

# 20. Items Not Promised for First Milestone

GNSS-SDR.

GPS-SDR-SIM.

GMAT.

Orekit.

TWSTFT.

CCSDS packet timing.

Transponder model.

Real Rubidium hardware unless already available.