Spacecraft Timekeeping Final Unified Execution Plan

Pure Python simulation, tabletop timing demonstrator, RC-car spacecraft-bus demonstrator, and high-fidelity extensions

# 1. Purpose

This document is the single execution plan for the spacecraft timekeeping project. It keeps the project focused by using one architecture, one phase sequence, and one block-diagram format across the Functional Flow Report, Python Guide, Final Presentation, and RC-car proof of concept.

# 2. Core Rule

The RC car is not a separate architecture. It is a spacecraft bus analogue: a physical carrier for the timing avionics. The same timing system is first proven in pure Python, then on a tabletop with real GNSS hardware, then mounted on the RC car.

Use the same block diagram everywhere.

Use phase names instead of version names.

Keep GNSS-SDR, GPS-SDR-SIM, GMAT, Orekit, TWSTFT, CCSDS, and transponder modeling out of the first milestones.

Use a software Rubidium model until a real clock source is available.

# 3. Frozen Block Diagram

This block diagram is the master architecture. For the RC-car demonstrator, the RC car replaces the satellite bus as the carrier platform.

GPS Satellites ¦ v GNSS Antenna ¦ v TR-2S Receiver ¦ v GNSS Time + PPS ¦ v Raspberry Pi / Mini PC ¦ v Time Synchronization Software ¦ v Master Time Output ¦ v PPS Distributor (Software) ¦ v Simulated TMTC / AOCS / Payload / OBC ¦ v Telemetry ¦ v Laptop Dashboard Physical carrier for hardware demonstrator: RC Car Chassis

# 4. Phase Plan

| Phase | Name | Goal | Main Work | Output |
| --- | --- | --- | --- | --- |
| Phase 1 | Pure Python Timing Simulation | Prove GNSS + Rubidium + Kalman filter = stable Master Time. | Rb clock model; simulated GNSS measurements; Kalman synchronization; software PPS distribution; Ground UTC comparison. | Four plots and a working Python simulation. |
| Phase 2 | Tabletop Timing Demonstrator | Prove the architecture with real GNSS data without moving the platform. | TR-2S receiver; GNSS antenna; Raspberry Pi; GNSS data reader; PPS monitoring; dashboard server. | Live dashboard showing GNSS status, PPS lock, time offset, and holdover behavior. |
| Phase 3 | RC-Car Spacecraft-Bus Demonstrator | Mount the same timing avionics on a mobile carrier and observe real GNSS behavior during motion. | RC car chassis; battery; Pi; TR-2S; antenna; logger; dashboard telemetry over WiFi. | Mobile demonstration with satellite visibility, GNSS quality, PPS stability, and time offset logs. |
| Phase 4 | High-Fidelity Timing Model | Add research-grade fidelity after the core architecture is proven. | GNSS-SDR; GPS-SDR-SIM; GMAT/Orekit; TWSTFT; CCSDS packet timing; transponder modeling. | Advanced validation model and research extension. |

# 5. Phase 1 Repository Structure

spacecraft_timekeeping/ rb_clock.py gps_receiver.py kalman_clock.py pps_distributor.py main_simulation.py plots/

# 6. Phase 1 Python Modules

| Module | Responsibility | Minimum Output |
| --- | --- | --- |
| rb_clock.py | Generate Rubidium clock bias, drift, and drift-rate over 24 hours. | Raw clock error time series. |
| gps_receiver.py | Generate simulated GNSS timing measurements using true time plus Gaussian noise. | Noisy GNSS measurement time series. |
| kalman_clock.py | Fuse Rubidium prediction and GNSS measurements into corrected Master Time. | Corrected clock error time series. |
| pps_distributor.py | Apply per-subsystem PPS delay and jitter to TMTC, AOCS, Payload, and OBC. | PPS arrival time and skew values. |
| main_simulation.py | Run the complete Phase 1 simulation and save plots. | Four plots and printed validation summary. |

# 7. Phase 1 GNSS Model

Use a simple measurement model first. Do not use GNSS-SDR or GPS-SDR-SIM in Phase 1.

gps_time = true_time + np.random.normal(0, 30e-9)

# 8. Required Phase 1 Plots

| Plot | Title | Purpose |
| --- | --- | --- |
| 1 | Raw Rubidium Drift | Show how the onboard clock drifts without correction. |
| 2 | Kalman Corrected Error | Show improvement after GNSS correction. |
| 3 | PPS Skew | Compare timing received by TMTC, AOCS, Payload, and OBC. |
| 4 | Ground UTC Offset | Show spacecraft time error against the ground reference. |

# 9. Phase 2 Tabletop Timing Demonstrator

The first hardware milestone must stay on a table. This proves the architecture without vehicle motion, vibration, mounting issues, or field-test complexity.

| Hardware | Role |
| --- | --- |
| TR-2S receiver | Provides GNSS time and PPS. |
| GNSS antenna | Receives satellite signals. |
| Raspberry Pi 5 or mini PC | Runs synchronization software, logger, and dashboard server. |
| Battery pack or stable power supply | Powers the receiver and Pi. |
| Laptop dashboard | Displays GNSS status, PPS lock, Master Time error, and mode. |

# 10. Phase 3 RC-Car Spacecraft-Bus Demonstrator

After the tabletop demonstrator works, mount the same hardware stack on the RC car. The car represents the spacecraft bus only. It carries the avionics and allows motion-based GNSS observation.

| RC-Car Demonstrator Element | Meaning in the Project |
| --- | --- |
| RC car chassis | Physical bus carrying avionics. |
| TR-2S receiver + antenna | GNSS timing source. |
| Raspberry Pi / mini PC | Onboard computer. |
| Software Rubidium model | Holdover clock model until real Rubidium is available. |
| Simulated TMTC / AOCS / Payload / OBC | Subsystem timing consumers. |
| Laptop dashboard | Ground monitoring station. |

# 11. Dashboard Demonstration Script

| State | Action | Dashboard Output |
| --- | --- | --- |
| Normal | Antenna connected and satellites visible. | GNSS visible; PPS locked; Master Clock Error in ns range. |
| Holdover | Unplug or shield antenna. | GNSS lost; Mode = Holdover; Rubidium model active; clock error slowly increases. |
| Recovery | Reconnect antenna. | GNSS acquired; resynchronizing; PPS locked. |

# 12. Hardware Responsibilities on Raspberry Pi

GNSS data reader.

Kalman filter.

PPS distribution logic.

Logger.

Dashboard server.

Optional hardware such as IMU, camera, telemetry radio, and real Rubidium should remain outside the first hardware milestone.

# 13. Weekly Execution Sequence

Freeze the block diagram and use it in every document.

Implement rb_clock.py and gps_receiver.py and generate the raw drift plot.

Add kalman_clock.py and generate raw versus corrected error.

Add pps_distributor.py and generate the PPS skew plot.

Add Ground UTC comparison and finish the four Phase 1 plots.

Move the code to Raspberry Pi for tabletop testing.

Read TR-2S data and PPS status on the Pi.

Run the dashboard from the Pi and view it on the laptop.

Mount the same system on the RC car after tabletop testing is stable.

# 14. Literature Reference Alignment

| Project Element | Key References | Reason |
| --- | --- | --- |
| Rubidium / clock model | R1, R2, R5 | Clock noise, drift, and Allan deviation validation. |
| GNSS timing and PPS | R6, R10, R11, R12 | GNSS timing behavior, PPS measurement, and receiver timing context. |
| Kalman synchronization | R13, R15, R17, R18 | Clock bias/drift estimation and process-noise modeling. |
| PPS distribution | R6, R7, R8, R9 | PPS accuracy, onboard time access, and mission timekeeping architecture. |
| Ground UTC validation | R30, R31, R32 | UTC handling, ground comparison, and ISRO/NavIC timing context. |
| Phase 4 extensions | R19-R29 | TWSTFT, transponder, CCSDS, TMTC, and AOCS timing extensions. |

# 15. CEO Milestone Statement

Milestone 1 demonstrates GNSS/Rubidium synchronization, Kalman correction, PPS distribution, and Ground UTC comparison in pure Python. Milestone 2 demonstrates the same architecture on a tabletop using real GNSS and a Raspberry Pi dashboard. Milestone 3 mounts the proven hardware stack on an RC car to show mobile GNSS behavior and live telemetry.

# 16. Items Removed From Early Milestones

GNSS-SDR.

GPS-SDR-SIM.

GMAT.

Orekit.

TWSTFT.

CCSDS packet timing.

Transponder model.

Real Rubidium hardware unless already available.