Spacecraft Timekeeping Unified Execution Plan

Phase-focused plan for pure Python simulation, functional flow alignment, and milestone communication

# 1. Purpose

This document freezes the project direction and converts the literature survey, functional flow report, and Python guide into one executable plan. The immediate objective is to deliver a credible Phase 1 simulator that proves GNSS plus Rubidium plus Kalman filtering can generate stable Master Spacecraft Time and distribute PPS timing to onboard subsystems.

# 2. Immediate Scope Decision

Phase 1 must stay small enough for one person to implement and defend. The first milestone is not a GNSS receiver, ground segment, flight software, or space communications project. It is a timing synchronization simulator.

| Keep in Phase 1 | Move out of Phase 1 |
| --- | --- |
| Rubidium clock noise model | GMAT |
| Simulated GNSS time measurements | GPS-SDR-SIM |
| Kalman clock synchronization engine | GNSS-SDR |
| PPS distribution and skew model | TWSTFT |
| Ground UTC comparison using simple reference time | CCSDS packet timing |
| Four validation plots | Orekit |
| Pure Python repository structure | Transponder model |

# 3. Frozen Architecture

This architecture should appear identically in the Functional Flow Report, Python Guide, and Final Presentation.

GNSS / TR-2S ¦ v Time Synchronization Engine ^ ¦ Rubidium Clock ¦ v Master Spacecraft Time ¦ v PPS Distribution System ¦ +---+---+-------+ v v v v TMTC AOCS Payload OBC ¦ v Time Telemetry ¦ v Ground UTC Validation

# 4. Phased Development Plan

| Phase | Goal | Included Work | Excluded Until Later |
| --- | --- | --- | --- |
| Phase 1: Pure Python Timing Simulation | Prove GNSS + Rubidium + Kalman Filter = Stable Master Time. | Rb clock model; simulated GNSS measurements; Kalman synchronization; PPS distribution; Ground UTC comparison. | GNSS-SDR, GPS-SDR-SIM, GMAT, Orekit, TWSTFT, CCSDS, transponder. |
| Phase 2: Ground Validation | Show how spacecraft time is checked from the ground. | Propagation delay; one-way time transfer; Ground UTC comparison; holdover analysis. | Full SDR receiver chain and flight-software packet implementation. |
| Phase 3: High-Fidelity Timing Model | Increase realism after Phase 1 is stable. | GNSS-SDR; GPS-SDR-SIM; GMAT/Orekit; TWSTFT; CCSDS packet timing. | Anything not required to prove the timing concept. |

# 5. Phase 1 Repository Structure

spacecraft_timekeeping/ rb_clock.py gps_receiver.py kalman_clock.py pps_distributor.py main_simulation.py plots/

# 6. Phase 1 Module Definitions

| Module | Responsibility | Minimum Output |
| --- | --- | --- |
| rb_clock.py | Generate Rubidium clock bias, drift, and drift-rate over a 24-hour simulation. | Raw clock error time series. |
| gps_receiver.py | Generate simulated GNSS timing measurements using true time plus Gaussian noise. | Noisy GNSS measurement time series. |
| kalman_clock.py | Fuse Rubidium prediction and GNSS measurements into corrected Master Spacecraft Time. | Corrected clock error time series. |
| pps_distributor.py | Apply per-subsystem PPS cable delay and jitter to TMTC, AOCS, Payload, and OBC. | PPS arrival time and skew values. |
| main_simulation.py | Run the complete 24-hour Phase 1 simulation and save plots. | Four plots and printed validation summary. |

# 7. GNSS Measurement Model for Phase 1

Do not use GNSS-SDR or GPS-SDR-SIM in Phase 1. Use a direct timing measurement model:

gps_time = true_time + np.random.normal(0, 30e-9)

This is enough to test the synchronization engine. The standard deviation of 30 ns can be adjusted during sensitivity testing, but the first version should remain simple.

# 8. Required Phase 1 Plots

| Plot | Title | X Axis | Y Axis | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Raw Rubidium Drift | Time | Clock Error | Show how the onboard clock drifts without correction. |
| 2 | Kalman Corrected Error | Time | Clock Error | Show improvement after GNSS correction. |
| 3 | PPS Skew | Subsystem | PPS Arrival Error | Compare timing received by TMTC, AOCS, Payload, and OBC. |
| 4 | Ground UTC Offset | Time | Spacecraft Time - Ground UTC | Show final spacecraft time error against ground reference. |

# 9. This Week's Execution Plan

Freeze the architecture. Do not add new blocks until Phase 1 plots are generated.

Create the spacecraft_timekeeping repository with the five Phase 1 Python modules and a plots folder.

Implement rb_clock.py and gps_receiver.py only. Generate the first graph: clock drift over 24 hours.

Add kalman_clock.py. Generate raw error versus corrected error.

Add pps_distributor.py. Generate the PPS skew plot.

Add Ground UTC comparison in main_simulation.py and save all four plots.

# 10. Functional Flow Document Update

Replace the current development sequence with the following phased sequence:

Phase 1: Pure Python Timing Simulation - Rubidium clock model, simulated GNSS measurements, Kalman synchronization engine, PPS distribution simulation, and Ground UTC comparison.

Phase 2: Ground Validation - propagation delay, one-way time transfer, Ground UTC comparison, and holdover analysis.

Phase 3: High-Fidelity Timing Model - GNSS-SDR, GPS-SDR-SIM, GMAT/Orekit, TWSTFT, and CCSDS packet timing.

# 11. Python Guide Update

The Python guide should become a Phase 1 implementation guide first. Move advanced sections into an appendix or Phase 3 chapter.

| Current Content | Action |
| --- | --- |
| GNSS-SDR and GPS-SDR-SIM setup | Move to Phase 3 appendix. |
| GMAT/Orekit trajectory generation | Move to Phase 3 appendix. |
| TWSTFT and transponder model | Move to Phase 3 appendix. |
| CCSDS packet timing | Move to Phase 3 appendix. |
| SimulatedGPSReceiver | Promote to main Phase 1 GPS model. |
| main_simulation.py | Make it run without external tools. |

# 12. CEO-Facing Milestone Statement

Milestone 1, planned for 4 to 6 weeks, will demonstrate GNSS/Rubidium synchronization, Kalman correction, PPS distribution, and Ground UTC comparison in pure Python. This milestone proves the timing concept and produces validation plots before introducing GNSS hardware, SDR processing, two-way transfer, or flight software integration.

Milestone 2, planned as an additional 4 to 6 weeks after Milestone 1, will connect real GNSS hardware or higher-fidelity timing models and expand the ground validation layer.

# 13. Success Criteria for Phase 1

The simulation runs from a clean Python environment using only standard scientific Python packages.

The Rubidium clock error plot shows realistic drift over 24 hours.

The Kalman-corrected error is visibly smaller than the raw Rubidium error.

The PPS skew plot shows distinct timing offsets for TMTC, AOCS, Payload, and OBC.

The Ground UTC comparison plot shows final spacecraft time error over the simulation period.

The Functional Flow Report, Python Guide, and Final Presentation all use the same frozen architecture and phase labels.

# 14. RC Car Proof-of-Concept Extension

The RC car should be treated as a spacecraft bus analogue. The car is not the project focus; it is only a mobile carrier for the timing avionics. This keeps the demonstration understandable: the same timing architecture can run on a desktop first, then later be mounted on a moving platform.

| Spacecraft-Term Concept | RC-Car Demonstrator Equivalent |
| --- | --- |
| Spacecraft bus | RC car chassis carrying avionics. |
| GNSS antenna | GNSS antenna mounted on the car or table. |
| Space-qualified GNSS receiver | TR-2S receiver used as the timing receiver. |
| Onboard computer | Raspberry Pi 5 or mini PC. |
| Time synchronization engine | Python synchronization software running on the Pi. |
| PPS distribution system | Software PPS distributor and monitoring logic. |
| TMTC, AOCS, Payload, OBC | Simulated subsystem processes. |
| Ground station / MOC display | Laptop dashboard over WiFi. |

# 15. RC-Car Demonstrator Architecture

GPS Satellites ¦ v GNSS Antenna ¦ v TR-2S Receiver ¦ v GNSS Time + PPS ¦ v Raspberry Pi / Mini PC ¦ v Time Synchronization Software ¦ v Master Time Output ¦ v PPS Distributor (Software) ¦ v Simulated TMTC / AOCS / Payload ¦ v Telemetry ¦ v Laptop Dashboard Mounted on: RC Car Chassis

# 16. Demonstration Versions

| Version | Setup | Purpose | Do Not Add Yet |
| --- | --- | --- | --- |
| Version 1: Desktop Timing Demonstrator | TR-2S, GNSS antenna, Raspberry Pi, dashboard, and PPS monitoring on a table. | Prove the architecture with the lowest risk. The car does not move. | IMU, camera, telemetry radio, vehicle motion, hardware Rubidium. |
| Version 2: Mobile Timing Demonstrator | Mount the same system on an RC car and drive in a parking lot, campus, or office area. | Observe real GNSS visibility, GNSS quality, PPS stability, time offset, and telemetry while moving. | High-fidelity space propagation, TWSTFT, CCSDS flight packet implementation. |

# 17. Hardware and Pi Responsibilities

| Required Hardware | Optional Hardware |
| --- | --- |
| Raspberry Pi 5 or mini PC | IMU |
| TR-2S receiver | Camera |
| GNSS antenna | Telemetry radio |
| Battery pack | Real Rubidium source |
| WiFi dongle or onboard WiFi | RC car mount vibration isolation |

The Raspberry Pi should run only the following services in the first hardware proof of concept:

GNSS data reader.

Kalman filter.

PPS distribution logic.

Logger.

Dashboard server.

# 18. Demonstration Script

| Scenario | Operator Action | Dashboard Should Show |
| --- | --- | --- |
| Normal locked operation | Run the system with antenna connected and satellites visible. | GPS Satellites Visible = around 8 to 12; Master Clock Error = single-digit to tens of ns; PPS Status = Locked. |
| Simulated GNSS failure | Unplug or shield the GNSS antenna. | GNSS Lost; Mode = Holdover; Rubidium model active; clock error slowly increasing. |
| Recovery | Reconnect or uncover the antenna. | GNSS Acquired; Resynchronizing; Locked. |

If a real Rubidium source is not available, Phase 1 and the first hardware demonstration should use the software Rubidium model. The demonstration goal is to prove the synchronization logic, failure handling, and dashboard behavior.

# 19. RC-Car POC Timeline

Begin this only after the Phase 1 simulator produces the four required plots.

| Week | Target |
| --- | --- |
| Week 1 | Move Phase 1 code to Raspberry Pi. |
| Week 2 | Read TR-2S GNSS receiver data and PPS status. |
| Week 3 | Run dashboard from the Pi and view it on the laptop. |
| Week 4 | Mount the system on the RC car chassis. |
| Week 5 | Field test GNSS visibility, PPS stability, time offset, and telemetry. |

# 20. CEO Demonstration Proposal

| Demo | Message | Risk Level |
| --- | --- | --- |
| Demo A: Desktop Timing Demonstrator | Real GNSS plus synchronization engine plus dashboard. This is the fastest and lowest-risk proof that the timing architecture works. | Low |
| Demo B: Mobile Timing Demonstrator | Same timing system mounted on an RC car. This shows real motion, real GNSS data, and live telemetry. | Medium |

Do not promise GNSS-SDR, TWSTFT, GMAT, Orekit, CCSDS, or a real Rubidium source for the first CEO milestone. Promise a working timing demonstrator first, then expand fidelity after the architecture is proven.

# 21. Literature Survey Reference Alignment

The RC-car demonstration is supported by the literature at the subsystem level. The references justify the timing logic, GNSS timing, PPS distribution, clock modeling, Kalman correction, and UTC validation. They do not claim that an RC car is a spacecraft; the RC car is a practical carrier used to demonstrate avionics behavior.

| POC Element | Literature Support | How It Is Used |
| --- | --- | --- |
| Rubidium / clock model | R1 Khatri 2014; R2 Vannicola 2003; R5 Allan 1987 | Supports clock noise, drift, and Allan deviation validation for the software Rubidium model. |
| GNSS timing and PPS input | R6 Winternitz 2019; R10 Hwang patent; R11 Bauer 1999; R12 Forero 2023 | Supports use of GNSS time, 1PPS measurement, receiver timing behavior, and software emulation path. |
| Kalman synchronization engine | R13 Hauschild and Montenbruck 2019; R15 Yang 2023; R17 Zhang 2012; R18 Bhatt 2024 | Supports clock bias/drift estimation and process-noise modeling from clock dynamics. |
| PPS distribution and subsystem timing | R6 Winternitz 2019; R7 CCSDS 872.0-M-1; R8 Cooper 2006; R9 Cooper 2012 | Supports PPS accuracy, onboard time access, mission timekeeping, and verification concepts. |
| Ground UTC validation and failure/recovery logic | R30 Nelson 2001; R31 Li 2015; R32 ISRO/SAC 2023 | Supports UTC/leap-second awareness, ground-to-satellite synchronization concepts, and India/ISRO timing context. |
| Advanced Phase 3 items | R19-R23 for TWSTFT/transponder; R24-R26 for CCSDS/TMTC; R27-R29 for AOCS timing | Kept out of Phase 1 and RC-car Demo A; used later when adding higher-fidelity timing and subsystem realism. |